SYCLOP LP18 DMX

High lumen output and low energy consumption framing projector.

Focusabe optical zoom, beam angle from 18° to 42°.

Framing by 4 quick-and-easy-positioning shutters.

Light Leaks free.

Tool free, quick-and-easy removal of the optical rotating barrel to convert it into a spotlight.

Tilt angle 200° - azimuth 365°.

Tool free access to the lenses. Holding positions by friction and locking screw. Handle free.

Dimming control by DMX 512 signal.

RDM addressing or defined in the factory.

Factory configurations:

• 8 or 16 bits

• Linear dimming curve / Smoothed Linear curve / Squared curve / Logarithmic curve

• Start-up Power percentage without DMX signal.

• Percentage or last value in case of loss of DMX signal.

Input signal by Data track adaptor or XLR 5 in/out / RJ 45 in/out.

230 Volts power supply : 3 circuits track adaptor Global Pulse or Eutrac Data, or Powercon in/out for the hook mounting version.

Integrated Gobo holder and filter holder.

Hook mounting version: supplied with TRUE 1 power cable and plug 2P+E, length 1 meter.